MICROCONTROLLER CONTROL of an ELECTRO-PNEUMATIC MANIPULATOR

Dr. D.E. Ventzas	           Tsitsipis P.		Kalovrektis K.	          Varzakas K

Professor Control Eng.    Assistant Professor	Graduate Student        Scientific Assistant

TEI Lamia - Lamia          Electronics-TEI Lamia	Electronics Dpt	          Electronics-TEI Lamia

35 100 - GREECE	          35 100 - GREECE 	TEI Lamia-Lamia       35 100 - GREECE

ABSTRACT:	The paper designs, implements and tests a 3 arms

electropneumatic polar manipulator of 3 dof, suitable for

educational purposes and CNC tool changers. Its controller

and its structure are presented.

KEYWORDS:	Manipulator,  microcontroller, control,

tool changer.

INTRODUCTION:	Materials transport and handling under harsh environments, lead to increased and specialized use of robots (Craig, J. J, 1989, Fu, K. et al, 1987). The paper designs and implements a microcontroller -based controller for manipulators.

The design, see fig. 1, consists of the electro-pneumatic manipulator, the microcontroller, see fig. 2, and the interface drivers, see fig. 5 (Kalovrektis C, 1996).

MICROCONTROLLER					SOLENOID		 PNEUMATIC

MC68HC11		DRIVERS	  	 VALVES	         MANIPULATOR

Electropneumatic interface

INTERFACE

OPTIMAL

IBM PC		SOFTWARE		CONTROL		PRODUCTIVITY

Problem definition

Fig.  1.   Block diagram of pneumatic manipulator control

THE MICROCONTROLLER:	The primary differences among the members of the M6811 family are the type and quantity of memory; CMOS technology offers high density, high data transfer speed, low power level (increased under STOP /WAIT mode control) and noise immunity. The MC68HC11A1, see fig. 3,  presents all the 6801 and 6805 MCU features (except for the PLL of the 6805), i.e it contains on-chip peripherals of the family and it is user programmable on various levels, see Table 1.

| MC68HC11A1 MICROCONTROLLER |  |  |  |  |  |  |  |  |  |  |  | CHARACTERISTICS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ROM |  |  |  |  |  |  |  |  |  |  |  | 8 kbytes |
| RAM |  |  |  |  |  |  |  |  |  |  |  | 256 bytes |
| EEPROM |  |  |  |  |  |  |  |  |  |  |  | 512 bytes |
| ADCs | 8 channels-8 bits with register ADCTL (CCF, SCAN, MULT, CD, CC, CB, CA) |  |  |  |  |  |  |  |  |  |  |  |
| Digital Input / Output lines |  |  |  |  |  |  |  |  |  |  |  | 22 |
| Ports |  |  |  |  | A, B, C, D (for communication), E (analog) (all 8 bits) |  |  |  |  |  |  |  |
| Data Direction Registers (DDR) |  |  |  |  |  |  |  |  |  |  |  | DDRC, DDRD |
| Asynchronous Communication |  |  |  |  |  |  |  |  |  |  | Serial Communication Interface (SCI) |  |
| Synchronous Communication |  | Serial Peripheral Interface (SPI) with lines MISO,MOSI, SCK, SS, register SPCR(SPIE,SPE,DWOM,MSTR, CPOL, CPHA, SPR1, SPR2) register SPSR (SPIF, WCOL, MODF), registers SPDR |  |  |  |  |  |  |  |  |  |  |
| Timer |  |  |  |  |  |  |  |  |  |  |  | 8 bits |
| Pulse Counter |  |  |  |  |  |  |  |  |  |  |  | timer independent |
| Control registers |  |  |  |  |  |  |  |  |  |  |  | IX, IY, SP, PC, CCR |
| Accumulators |  |  |  |  |  |  | 8 bits A, B accumulator, 16 bits A+B, Pulse accumulator |  |  |  |  |  |
| Condition Code Register |  |  |  |  |  |  |  | C/B, O, N, I, H, Interrupt Mask (X), Stop Disable |  |  |  |  |
| Control lines |  |  |  |  |  |  |  |  |  |  |  | 3 |
| Control output lines |  |  |  |  |  |  |  |  |  |  |  | 5 |
| Watch Dog Timer |  |  |  |  |  |  |  |  |  |  |  | yes (COP) |
| Commands repertoire |  |  |  |  |  | over 150 (90 new) opcodes (move, arithmetic, logic, edit, control) |  |  |  |  |  |  |
| Clock |  |  |  | XTAL - EXTAL 8 MΗz (internally divided to 2 MHz), down to DC |  |  |  |  |  |  |  |  |
| Interrupts |  |  |  |  |  |  |  |  | IRQ, XIRQ (MI, NMI, RTI) with HPRIO register |  |  |  |
| INIT register |  |  |  |  |  |  |  |  |  |  |  | redefines RAM area |
| Memory extension under |  |  |  |  |  |  |  |  |  |  |  | STRA, STRB |
| Modes of Operation |  |  |  |  |  |  |  |  |  | MODA, MODB (MUX), Bootstrap, Test, Security |  |  |
| Address Modes |  |  | Direct, Indirect, Extension, Indirect X, Y, Inherent, Relative |  |  |  |  |  |  |  |  |  |

* OTP (One Time Programmable)

Table  1. MC68HC11A1 microcontroller specifications (Hintz, K, et al, 1992)

ELECTRO-PNEUMATIC MANIPULATOR:		The manipulator is a polar mechanical structure with 3 degrees-of freedom, see fig. 4; it consists of components:

| TYPE OF PNEUMATIC COMPONENT | TECHNICAL CHARACTERISTICS |
| --- | --- |
| double acting pneumatic cylinder | NA |
| single acting (spring-returned) cylinder | NA |
| rotary valve (0 - 180o) | NA |
| 5/2 and 2/2 electric valves (returned) | 10¸40 ms, 1.8¸8 bar, 500¸1000 l/min |
| vacuum pump (sucking valve control) | 6 bar, -660 mmHg, 20¸68 l/min |
| air pump | 10 bar |
| adjusting,relief valves,orifices, throttles | common |

that operate at a pressure of  3 bar. The robot gripper is of the vacuum type with one suction cup, (Rivin, E, 1988).

Fig.    2.  The microprocessor interface - controller system (Kalovrektis C, 1996)

___

MODA	MODB				IRQ	____	______

(LIR)	(VSTBY)	     XTAL  EXTAL   E	(VPP)	XIRQ	RESET

MODE CONTROL	OSC			INTERRUPT

CLOCK LOGIC		    LOGIC		ROM 8 kBytes

TIMER

SYSTEM							EEPROM 512 Bytes

COP			CPU CORE								    VDD

RAM 256 BYTES 	    VSS

OC1

PULSE			BUS EXPANSION	ADDRESS DATA

ACCUMULATOR		      ADDRESS		R/W  AS	PERIPHERAL	   SERIAL	      SERIAL

COMMUNICATION

INTERFACE	    INTERFACE

SPI		          SCI

PA1 OC2 OC3 OC4 OC5 IC1 IC2 IC3	    	   STROBE AND HANDSHAKE

PARALLEL I/O

STRB   STRA

CONTROL           CONTROL			ADC

PORT A		PORT B		   PORT C	    PORT D

PORT E

_

PA7..............PA0      PB7...............PB0	  PC7......PC0	 STRB/R/W STRA/AS PD5......PD0         PE7....PE0

Fig. 3. MC68HC11A1 microcontroller block diagram and memory map (Motorola, 1992)

Fig.   4.   The electro-pneumatic manipulator

MANIPULATOR MICROCONTROL:		We use 2-out-of 4 ports (the rest are for future use. Port B drives (not immediately, but through the CD4050 buffer) leds and power transistors for electropneumantic valves; this introduces a time delay between CD4050 input and output command transfer of 30 ns. The power transistors are on/off switches of 3 A, 30 W, 3 MHz (only 0.7 A, 12 V needed).

| MANIPULATOR | MOTION |
| --- | --- |
| Elbow rotation | clockwise / anti-clockwise |
| Elbow linear motion | opening / closing (in length) |
| Elbow linear motion | up / down |
| End effector | sucking (hold/leave) |

The electro-pneumatic interface combines digital and analog electronics logic gates, timers, limit switches and sensors with pneumatic logic and control valves.

MICROCONTROLLER SOFTWARE:		The software consists of: 		Drive commands			Loading commands

Delay (jump) commands		Menu (selection commands)

Pneumatic elements control is performed via port B, ($1007) i.e:

| TYPE OF VALVE | CONTROL via PORT B |
| --- | --- |
| r-linear valve | b3 |
| Rotary valve | b2 |
| z-linear valve | b0 |
| Sucking (handling) valve | b1 |

Pneumatic valves present a time delay in their response (FESTO, 1990); the software is delayed (by JSR variable delay) and feedback from limit switches (by BNE feedback); PAUSE is also delaying manipulation (Snyder,W, 1985, Spong,M. et al,1989, Tou,J,1985).

double acting cylinder					rotary cylinder

return						    return

spring						    spring

valve 1						valve 2

P					pressure reducers

single acting							vacuum pump

cylinder with					      R

return spring

adjusting

relief

valve

valve 2/2					         P

P					        P

manometer

2-4 bar

valve 2/2	P

adjusting

R   relief

valve		      compressor		      Pmax = 5 bar

air pump

Fig.   5.   Electro-pneumatic circuit for manipulator control

The pneumatic control problem to design the manipulator proceeds according to the following stages. Define and/or draw the:

| PROBLEM | ANSWER |
| --- | --- |
| task | tool change, etc |
| forces | < 4500 kp |
| stroke lengths | < 2000 mm |
| working speed | 15 ¸ 300 mm/s |
| location plant | simple |
| motion diagram | xyz time sequence |
| circuit diagram | pneumatic circuit |

linear valve 1

rotary

valve

linear valve 2

A			B					    C			         D

working

envelope						   h

H			G				  	   F				E

a			b

Fig.  6.  The electro-pneumatic robot mechanism drawing and its working envelope

STANDARD PROCEDURES:		At start-up and reset, the electro-pneumatic manipulator might operate in an unpredictable way; this should be avoided for operating safety reasons. During start-up calibration is performed by a certain sequence (ABCDEFGH), that executes and follows all possible geometrical end points accessing  operations of the manipulator working envelope, see fig. 6. Reset restarts the microcontroller and any calibration start-up sequence.

KEYBOARD		COMPUTER			   RS232C

ROBOT ARM		      feedback 			MC68HC11A1

feedback 					              POWER

SUPPLY

PNEUMATICS		ELECTRO-VALVES		DRIVERS

Fig.  7.  The robot’s general control system block diagram

FEEDBACK - POSITION DETECTORS:	The position detectors, see fig. 8, are magnetically coupled to the valves pistons, on non-ferrous pneumatic cylinders. A sine wave is fed to the drive coil and it is detected (after being attenuated by 30 dB) on the pick-up coil, at an operating frequency 100 Hz; they present a response (settling) time of 25 ms (ADC conversion time is 20 μs). For a piston’s rod extension of 0 ¸ 20 cm an exponential response voltage output of 60 ¸ 30 mV results (Tsitsipis, P, 1990).

Fig.  8.  Electronic diagram of piston position detector (Feedback, 1983)

- MANIPULATOR TEST:		Most robots are programmed, led by the nose, or use for operators training, under primitive conditions and operating environment. Such a training test for both robots, software and operators is the “Tower of Hanoi” problem (Fu et al, 1987). Here an other test is proposed: Moving two cans (or cards) with a hand around an axis of symmetry, is a (simplification of an illegal) gambling game with cards (that any tourist visiting the center of Athens should avoid to participate, because he would certainly be a looser); this game is played with three cards (two black and one coloured) and two hands; the gambler should guess (locate) the position of the coloured card. The following tests are performed (Tzafestas, S, 1991, 1992, Ventzas D, 1995, 1997, Vucobratovic, M. et al, 1982, Walker, M. et al, 1996) :

a.	open loop control	b.	closed loop control 	c.	grip control

d.	sequential or simultaneous control of all 3 axes (z, r, ζ)

APPLICATION - TOOL CHANGER:		In a large CNC machine, tools are often exchanged, for machining flexibility and optimal performance. A tool change includes the following steps:

a.	move tool changer close to the tool case

b.	rotate tool case (chained)

c.	send tool changer to the corresponding tool

d.	lock required tool to the tool changer

e.	move tool changer close to the machining CNC end effector

f.	stop CNC

g.	send tool changer to the machining CNC end effector tool

h.	lock end effector tool to the tool changer (opposite side)

i.	rotate tool changer

j. 	lock new tool to the machining CNC end effector

k.	move tool changer to a park position

l.	start CNC

m.	park old tool (if needed) to the tool case and/or park tool changer

All the above actions can be performed by the designed electro-pneumatic manipulator, i.e. we designed an optimal CNC operation, tool changer, see fig. 9.

Fig.  9.  CNC with tool changer (a) and tool changer

ACCURACY CONSIDERATIONS:		The operational accuracy of the robot is investigated. This is affected by the execution time of program loops (delays cause overshoot in position) and the design accuracy of the pneumatic components. The robot accuracy is tested at extreme (maximal and minimal) positions, where it is seeking the ideal test point.

CONCLUSIONS:		An electro-pneumatic manipulator has been designed and implemented under the scope of tools changing.

APPENDIX 1: 	TECHNICAL SPECIFICATION of  MC68HC11A1  -  IC  pins

| TECHNICAL SPECIFICATION of MC68HC11A1 - IC PINS DESIGNATIONS |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | vSS | 14 | PC5/A5/D5 | 27 | PA7/PA/OC1 | 40 | PB2/IA10 |
| 2 | MODB/VSTBY | 15 | PC6/A6/D6 | 28 | PA6/OC2/OC1 | 41 | PB1/IA9 |
| 3 | MODA/IIR | 16 | PC7/A7/D7 | 29 | PA5/OC3/OC1 | 42 | PB0/IA8 |
| 4 | STRA/AS | 17 | RESET | 30 | PA4/OC4/OC1 | 43 | PE0/AN0 |
| 5 | E | 18 | XIRQ | 31 | PA3/OC5/OC1 | 44 | PE4/AN4 |
| 6 | STRB/R/W | 19 | IRQ | 32 | PA2/IC1 | 45 | PE1/AN1 |
| 7 | EXTAL | 20 | PD0/RxD | 33 | PA1/IC2 | 46 | PE5/AN5 |
| 8 | XTAL | 21 | PD1/TxD | 34 | PA0/IC3 | 47 | PE2/AN2 |
| 9 | PC0/A0/D0 | 22 | PD2/MISO | 35 | PB7/IA15 | 48 | PE6/AN6 |
| 10 | PC1/A1/D1 | 23 | PD3/MOSI | 36 | PB6/IA14 | 49 | PE3/AN3 |
| 11 | PC2/A2/D2 | 24 | PD4/SCK | 37 | PB5/IA13 | 50 | PE7/AN7 |
| 12 | PC3/A3/D3 | 25 | PD5/SS | 38 | PB4/IA12 | 51 | VRL |
| 13 | PC4/A4/D4 | 26 | VDD | 39 | PB3/IA11 | 52 | VRH |

REFERENCES

- Craig, J. J, Introduction to Robotics, Mechanics and Control, 2nd Edition, Addison Wesley Pub. Co, 1989

- Feedback, Hydraulic Robot Arms, Type HRA933 & 934, Instruction Manual, 1983

- FESTO Pneumatic, Technical Data of Pneumatic Components, 1990

- Fu, K. S, Gonzalez, R. C, Lee C. S. G, Robotics: Control, Sensing, Vision and Intelligence, McGraw-Hill Book Co, 1987

- Hintz, K, Tabak, D, Microcontrollers, Architecture, Implementation and Programming, McGraw-Hill Inc, 1992

- Kalovrektis C, Manipulation Control of a Pneumatic Manipulator by an MC68HC11A1  Microcontroller, Diploma Thessis, Electronics Dpt, TEI Lamia, Lamia (in Greek), 1996

- Motorola, MC68HC11A1 Technical Manual, 1992

- Rivin, E, Mechanical Design of Robots, McGraw-Hill Inc, 1988

- Snyder, W. E, Industrial Robots: Computer Interfacing and Control, Prentice-Hall, Englewood Cliffs, New Jersey, 1985

- Spong, M.. W, Vidyasagar, M, Robot Dynamics and Control, New York, Wiley, 1989

- Tou, J. T, Computer-Based Automation, Plenum, New York, 1985

- Tsitsipis, P, Electronic Measurements, Lecture Notes, TEI Lamia,Dpt. Electronics Eng, Lamia, Greece, 1990 (in Greek)

- Tzafestas, S. G, Microprocessors in Robotics and Manufacturing Systems, Kluwer, Boston-Dordrecht, 1991

- Tzafestas, S. G, Robotics and Flexible Manufacturing Systems, Elsevier/Nort Holland, Amsterdam, 1992

- Ventzas D. E, Spatial Mechanisms and Robotics, Lecture Notes, University of Thessalia, Dpt. Mechanical Eng, Volos, Greece, 1995 (inGreek)

- Ventzas D.E, Robotics Theory (I) and Technology (II), Lecture Notes,TEI Lamia, Dpt. Electronics Eng, Lamia, Greece, 1997 (in Greek)

- Vucobratovic, M. Stokic, D, Control of Manipulation Robots: Theory and Application, Monograph, Springer-Verlag, 1982

Walker, M. W, Orin, D. E, Efficient Dynamic Computer Simulation of Robotic Mechanisms, Transactions of ASME, Journal of Systems Measurement and Control, vol.104, pp.205-211

D.Ventzas, Analipseos 124, Volos, 38221, Greece - Fax: (30) (231) 33945- E-mail:dventzas@uth.gr