MOISTURE CONTROL OF COUNTER CURRENT FLOW DRYERS

Dr. D. E. Ventzas

Professor of TEI Lamia

Control & Instrument Eng.

ABSTRACT:		The paper presents a counter current

flow rotary dryer model for heat transfer and for humidity

control; dryer state variables functions and relations are given.

Control algorithms such as single or multiple control variables

and conventional or model-based and intelligent control

schemes are investigated. Simulation results are given.

KEYWORDS:	Dryer, moisture, control, quality

model, turbulence, simulation.

INTRODUCTION:		Drying and humidifying processes present a variety of types (rotary, with fins and fluidized beds) that correspond to the control and mechanical solutions, with or without moving parts (Milg K. M, 1964, Barber, M, et al, 1995, Brook, R.C, 1987, Wedel, G. L, et al, 1987).

A counter current rotary dryer consists of a heater, a stationary compartment with its material feed and its sealing, the dryer with the cage, the dryer drive motor, a hopper, a cyclone as dust collector, a fan unit, the discharge unit with an exhaust fan and the material discharge exit, see fig. 1.

The paper presents the modeling, operation and control of a counter current flow dryer and the case of moisture control. It takes into account variable drying conditions and test control strategies(Morey, R.V, et al, 1979, Stroshine, R, et al 1992).

DRYER MODEL:		Theoretical study of dryers is difficult, so the overall unit drying efficiency and quality is derived experimentally (Allen R, et al, 1994), see fig. 2, 3, 4; combination of drying units and techniques (Lynch, B.E, 1989) results in higher efficiency and minimal start-up time, under a wide range of drying conditions and materials (Courtois F, et al, 1991). Heat transfer in rotary dryers is related to a power of 0.8 of the air mass velocity (Mykelestad, O, 1972). Heat and mass transfer relations are:

Uv=0.52.G0.8		NNu=0.2.NRe0.83			where,

Uv	=	volumetric heat transfer [kcal/hr.m3.oC]

G	=	air mass velocity [kg/hr.m2]

NNu	=	Nusselt number		NRe	=	Reynolds number

- external view			b.	operational diagrams

Fig.  1. 	Rotary-linear dryer - cooler (longitudinal installation)

kg/hr					     	mA

160					     	9

140					     	8    15%

120					     	7	       12%

100					     	6	             10%

80					     	5	                     9%

8%

60					     	4		               7%

6%

40	     sand mass flow [kg/hr] =			 			5%

= 18.2 * control signal + 18.0	     	2

20

1

0       1      2     3       4       5       6        7	    0      1       2       3       4       5        6      7

control signal	[mA]			sand control signal  [mA]

- sand flow (vibration screw feeder)              b.     percentage moisture curves

Fig.   2.  System parameters

volumetric coefficient

of heat transfer [kcal/hr.m3.oC]			time in dryer [min]

750

700						30

650

600

550						20

500

450

400						10

4000    5000    6000    7000     8000    9000		4000		6000		8000

mass air velocity  (kg/hr.m2)			mass air velocity  (kg/hr.m2)

Fig.  3.   Relations between dryer operating variables (Vikas Saksena, et al, 1997)

air temperature [oC] 				solid moisture [%]

50						35

45						30

40						25

35						20

30						15

16     	    24     	          32	   40	      0	    0.5           1.0         1.5         2.0

solid moisture	[%]	         	traveling length in dryer [m]

Fig.  4.  Relations between dryer operating variables

Air out										air in

dv

Solids in									solids out

l	        dl

Fig.  5.  Counter current rotary drying

The moisture as a function of dryer length, see fig. 5, is:

X=Xi.exp−k.Uv.Aλ.F.l+b−tsk.exp−k.Uv.Aλ.F.l−1		where,

X	=	solids moisture at distance l from feed entrance [ kg/kg ]

Xi	=	solids moisture at distance feed entrance [ kg/kg ]

Uv	=	volumetric heat transfer coefficient [ kcal/hr.m3.oC ]

ts	=	solids temperature		ta	=	air temperature

l	=	distance			ta 	=	k.X + b

F	=	feed rate [ kg/hr ]		λ	=	latent heat at ts [ kcal/kg ]

In case of multiple pass dryers higher efficiencies are possible.

There is a strong coupling between quality and drying models; models complexity is simplified by equations assumptions, while model based control prevents from quality degradation (destruction of fibers structures, etc).

The dryer measurements are:

1.	drying medium temperature		2.	cooling medium temperature

3.	temperature / humidity of processesd material at the input / output

4.	drum rotating speed				5.	product quality

6.	drying /cooling medium mass/velocity	7.	turbulence

8.	drying material particle size		9.	tilt and oscillation frequency

10.	operating monitoring (fans, collectors, etc)

The rotating drum has a length of 4 * D (D= drum diameter) ; apart of the drum’s inclination there is also the drum’s tilt (left-to-right motion); during material passage along the drum’s, the tilt and the varying width of the material under drying, optimizes the overall drying / cooling. Drum’s construction materials are stainless steel for sewage, aluminium for erosion strength, Monel for further resistance. The rotating mechanisms are chains with minimum service requirements. In fin dryers the transfer efficiencies of 200 BTU / hr / sq.ft / o F  in a total surface of 10 ¸ 15 m2  have been measured. Sometimes the processed material is enriched by salts in inert atmosphere. The dryers with fins are vulnerable to corrosion in sewage treatment, while cooling is achieved by water mixed or heat exchangers.

Fluidization results in fluids like materilas, that present properties similar with the fluids; suitable air flow velocity achieves optimal fluidization and drying, under mild turbulence and for suitable flue gas temperature; excess turbulence leads to non economic pumping operating characteristics of the system. Minimal drying material loss, is achieved by a cyclone for material accumulation.

Due to high temperatures, local operators protection barriers are needed. Short drying time and low drying temperatures avoid material thermal shock. Industrial drying examples, are sugar drying at  38 ο C, municipal wastes and lignite drying.

DRYER CONTROL:	Α model-based moisture sensing and control algorithm (Maier, D.E, et al, 1994) and the differential temperature method for dryer control senses and controls moisture inside dryers(Mittal, G.S, et al, 1981), are considered. The influence of design parameters and operating conditions and parameters (global, intermediate and local), including control laws on the products qualities are examined.

For continuous dryers it is based on the mathematical model:

M = K1 . ΔT . p - K2 / S . q		where,

M 	= 	product moisture

ΔT 	= 	temperature drop of hot air after contact with the wet product

S 	= 	production rate or dryer speed

K1 ,K2 	=	constants unique to a system and product

p, q 	= 	exponents unique to a system and product

Dryers are continuous, semi-continuous or batch; for batch dryers, dryer speed is converted to drying time, Δt,  and the model becomes:

M = K1 . ΔT . p - K3  . Δt . r

The differential temperature method (Moreira, R.G, et al, 1992) uses only two  temperature probes for sensing moisture inside-the-dryer, resulting in unit energy savings by automatically adjusting the hot side temperature to an idle when there is no product in the dryer. By automatically adjusting the hot side temperature with varying incoming moisture the differential temperature method also narrows the moisture content distribution (quality improvement), thus eliminating overdrying. The energy  savings could reach 35% reduction in fuel consumption per ton of dry product (Watson, S. B, 1997).

Drying control operation presents various types of dryer control schemes (Ventzas, 1997) and should be able to handle temperature and relative humidity control in various types of dryers, initiating typical procedures for starting up and shutting down a dryer. Dryer is an excellent example for research and training in  control theory state de-coupling.

Single-loop controller does not automatically adjust the heat to save energy, it is unable to adjust heat when product is absent (or changed) and to handle variations in incoming product moisture, wasting energy due to inadequate control while it does not eliminates the possibility of wet product.

Fig.   6a.  Dryer simulation scheme

material humidity [ % ]			 material temperature

15 %					        40 oC

0	1	2	3	4	5	 0	1	2	3	4	5     	min

(b)						(c )

Fig.   6b, c.  Dryer simulation results output responses

The non linearities involved in drying are dryer saturation (i.e. part of the dryer volume is unused), drying air saturation (i.e. part of the drying ability of the air is lost), variance in moisture distribution of the material under drying processing, etc. Relative errors in flow measurements are minimized (Ventzas, 1997), while experimental uncertainties are difficult to evaluate.

Model robustness is proved by control simulation, where control parameters do not require drastic correction during experimentation. Simulation predicts dryer degradation.

CONCLUSIONS:		The paper presented the modeling, operation and moisture control of a counter current flow dryer. A heat transfer and moisture model were given. Model-based and differential temperature control schemes are proposed.

Modern dryers with air recirculation present tempering zones, i.e. drying stages with  different temperatures. Multi-stage techniques allow moisture to diffuse to the surface, thus increasing the drying rate, while resting at high temperatures strongly influences quality. CAD techniques assist in dryer design, selection and operational simulation.

Fig.   7.   Rotary dryer test		Fig.   8.  Dryer heat exchanger

REFERENCES

1.	Allen, R. M., & Bakker, H. H. C, Spray Dryer Control Based on Online Particle Size

Analysis, Trans IChemE, 72(A2), 251-254, 1994

2. 	Barber, M. Kauppila, J-Ili, Ilvespaa, H, The Future of Drying, PITA Conference

"Water Removal '95", Conference Proceedings, 5/6 December 1995

3.	Brook, R.C, A look at Control of  in-bin Grain Drying Equipment. Paper No.

87-6037. ASAE, St. Joseph, MI, 1987

4.	Colliver, D.G., Peart, R.M., Brook, R.C., and Barrett Jr., J.R, Energy usage for low

temperature grain drying with optimized management, ASAE Transactions. 26:

594-600, 1983

5.	Courtois F., Lebert A., Duquenoy A., Lasseran J.C., Bimbenet J.J., Modeling of

Drying to Improve Processing Quality of Maize, Drying Technology, 9(4),

pp 927-945, 1991

6.	Lynch, B.E. and Morey, R.V, Control Strategies for Ambient Air Corn Drying,

ASAE Transactions, 32(5):1727-1736, 1989

7.	Maier, D.E., Dalpasquale, V.A., and Kallestad, D, High-Low Temperature Drying

plus Automatic Conditioning, Paper No. 94-6042. ASAE, St. Jospeh, MI, 1994

8.	Milg K. M., A Study of the Dynamics in a Rotary Drum Dryer with Counter

Current Flow, Chr. Michelesen Institute, Norway, Bergen 1964

9.	Mittal, G.S. and Otten, L, Evaluation of various Fan and Heater Management schemes

for Low - Temperature Corn Drying. Can. Agric. Engng. 23(2):97-100, 1981

10.	Moreira, R.G. and Bakker-Arkema, F.W, Grain dryer control: A review,  Cereal

Chem. 69(4):390-396, 1992

11.	Morey, R.V., Cloud, H.A., Gustafson, R.J., and Petersen, D.W, Management of

Ambient Air Drying Systems, ASAE Transactions. 22(6):1418-1425. 1979.

12.	Mykelestad, O, Heat and Mass Transfer in Rotary Dryers, Chemical Eng,

Process Symposium Series,  pp. 129-137, No. 41, vol 59, 1972

13.	Stroshine, R.L., Mc Kenzie, B.A., Sarwar, G., Thompson, T.L. and McFate, K.L.

Improving Management of  Low Temperature Drying of Corn, Technical Report

TR-100445. EPRI, Palo Alto, CA, 1992

14.	Ventzas, D, Control Theory, TEI Lamia, 1997 (in Greek)

15.	Vikas Saksena, Mike D. Montross, Dirk E. Maier, Site Specific Natural Air/Low

Temperature Drying  Strategies for White Food Corn, Department of Agricultural &

Biological Engineering, Purdue University, West Lafayette, Indiana, The ASAE

International Annual Meeting, The American Society of Agricultural Engineers,

Minneapolis, Minnesota, August 10-14, 1997

16. 	Watson, S. B, Improving Dryer Section Efficiency - A Scientific Approach, Paper

Technology 37, 6 July/August 1996

17. 	Wedel, G. L. and Palazzolo, S Advances in dryer section runnability, Tappi Journal

September 1987

APPENDICES

APPENDIX I :	   	SYSTEM SPECIFICATIONS

| S Y S T E M S P E C I F I C A T I O N S |  |  |  |  |
| --- | --- | --- | --- | --- |
| Dryer temperature set point accuracy |  |  | +/-5 °C |  |
| Ambient temperature controlled set points |  |  | with a 40 °C maximum |  |
| Individual set point control for each dryer |  |  | with a 45 °C maximum |  |
| Three system set point options | Fixed, Ambient plus, Ambient Humidity/Temperature |  |  |  |
| low temperature and high temperature alarms |  |  | customer selectable |  |
| Alarm output |  |  | 12V |  |
| Controller communication |  |  | lost alarms |  |
| Controls |  |  | up to 192 produce dryers |  |
| Daily log files | set points, ambient temperature, dryer temperature, |  |  |  |
| Log files |  |  | importable into any database or spreadsheet |  |
| Customer response control |  |  | frequency of temperature readings |  |
| Thermostat control |  | independently configured for each dryer controller |  |  |
| Remote system control |  |  | customer control of dryers |  |
| RS-485 interface bus computer/Network Controller |  |  |  | yes |

Dryer systems consist of a dryer controller, a weather station, a network controller and the dryer control system software. The dryer controller is a microprocessor based temperature controller used to control fan power and temperature of a produce dryer for increased temperature accuracy; the unit is a self contained thermostat controller that maintains the desired set point within +/- 10 °C. The weather station is a microprocessor based temperature sensor that is used to sense the temperature and humidity of the air being supplied to the produce dryer. The dryer control system software reads the temperature of the weather station and calculates the set point for the dryer controllers and monitors the controllers connected to the network. The Network controller is a microprocessor based communication station; this allows the dryer control system to support up to 8 networks of up to 24 dryer controllers (192 total). Energy savings and reduction in unit energy costs (Colliver, D.G, et al, 1983) are achieved by reducing wasting energy due to poor control, reducing flue gases going out the exhaust stack, stopping the dryer to run when no product is in the dryer, and defining a consistent target product moisture content without over-drying or wasting energy during startup or down times.

APPENDIX II : 	MATERIALS under PROCESSING (DRYING, COOLING, etc)

| MATERIALS under PROCESSING (DRYING, HUMIDIFYING, COOLING, etc) |  |  |  |  |
| --- | --- | --- | --- | --- |
| alumina | blood | glass | salts | biomass |
| foods | cheeses | rice | soap | flour |
| asbestos | cocoa | tobacco | powders | retines |
| medicine | coffee | lignite | sewages | cotton |
| boxite | grains | lime | sand | resins |
| bones | detergents | milk | sugar | popcorn |
| petfood | grains | plastics | chemicals | cement |

APPENDIX II : 	DRYING - COOLING  - HUMIDIFYING  METHODS

| EVALUATION of DRYING - COOLING - HUMIDIFYING METHODS |  |  |  |
| --- | --- | --- | --- |
| TYPE | ROTARY | with FINs | FLUIDIZED BED |
| OPERATION | D, H, D & H 1 | D, H, C | D, H, C |
| OPERATION | warm, humid air | fins/pipes exchange | vertical, warm air |
| CAPACITY | small - large | small - large | medium - very large |
| PROPERTIES of MATERIALS | free flow, products and specifications | thin with grains | small size |
| TYPICAL MATERIALS | foods, plastics, chemicals | food, sugar, coal, chemicals, etc | minerals, chemicals, coal, cock, etc |
| DRYING TIME | constant, variable, dust removal | constant, variable, dust collectors | short time, dust cyclones |
| CRITERIA | drying quality, variable times, temperatures | inert atmosphere, add oil and salts | large quantities, dry and cool simultaneous |

1 	D	=	Dryer	,	H	=	Humidifier	, 	C	=	cooling

D. E. Ventzas, Analipseos 124, Volos 382 21, Greece, Fax: 30-421 - 69787- E-mail:dventzas@uth.gr