MULTI-BODY DYNAMICS in COMPLEX OBJECTS CONTROL:

an INTERNAL COMBUSTION ENGINE DYNAMOMETER TEST

D. E. Ventzas					P. Tsiakaras

University of Thessaly - Visiting Res. Assistant	University of Thessaly

Analipseos 124, Volos, 38221, Greece,		Pedion Areos - 383 34

FAX: (0030) (231) 33945 				FAX: (0030) (421) 69787

E-mail:   dventzas@uth.gr			E-mail:   tsiak@uth.gr

The principles and the methods to control complex, multi-body, objects of diverse physical nature, subjected to complex forces and comprise inertial components having structural compliance under constraints are described. Non-linear control results in improved response, extended torque frequency range, system dynamics and overall speed. The paper presents the model and simulation of an internal combustion engine coupled with an eddy current dynamometer system. Steady-state and dynamic models are presented.

KEYWORDS

Multi-body, dynamics, control, test, internal combustion engine, dynamometer.

INTRODUCTION

The performance of an internal combustion engine is evaluated and a coupled eddy current dynamometer system is described (see fig. 1). Lumped modeling techniques are used in order to establish the inter-connections between the physical components. The coupled eddy current dynamometer absorbs kinetic power by adjusting its field excitation current.

Under steady state conditions, the engine is simulated as a voltage source with resistance, while the dynamometer is modeled as a current source with impedance. Under dynamic conditions the engine is simulated as a voltage source with resistance and capacitance, while the dynamometer is considered as a current source with impedance, capacitance and reactance. The shaft is a passive element with impedance-reactance characteristics. The shaft stiffness is measured physically, and the internal damping is derived by transient response analysis (1).

DYNAMOMETER SELECTION

Dynamometers are mechanical, hydraulic, electric, eddy current, air brake, friction and hybrid ones (9, 10). Eddy current dynamometers are capable of rapid load change, well adapted to computer control, simple and robust with low inertia, but they are vulnerable to poor cooling supply, not sustaining overload (11). Engine and dynamo-meter characteristics are matched (see fig. 3). An incorrect choice, installation and use of the system might lead to torsional oscillations, system vibration and shaft whirling and eventual failure. The resonant (critical frequency) of torsional oscillation is (2):

nc=60pC.(Iengine+Ibrake)Iengine.Ibrake

Throttle Angle								Field Current

ENGINE		COUPLING SHAFT	             DYNAMOMETER

Torque			       Speed

Measurement			   Measurement

Fig.  1.	Block diagram of the plant

Fig.  2.  Experimental flexible link

100%		       b			100 %				c

torque			         c		power		  b

a									d

d

e

e			        a

0		speed	     100 %	  0		speed	     100 %

Fig.  3.  Performance curves of eddy current dynamometer

For a three body model of a load-motor-tachometer system:

sLasKE00−kTs2Jm+sDs2J1s2J20sD1+k1−s2J1+sD1+k100sD2+k20−s2J2+sD2+k2.Ia(s)qm(s)q1(s)q2(s)=V(s)000

where,

La	=	motor equivalent inductance

kE	=	generator voltage / angular velocity proportionality constant

kT	=	motor torque / current proportionality constant

k1	=	load (brake) stiffness factor

k2	=	tachometer stiffness factor

Jm	=	motor moment of inertia

J1	=	load (brake) moment of inertia

J2	=	tachometer moment of inertia

D	=	damping factor

D1	=	load (brake) damping factor

D2	=	tachometer damping factor

Ia	=	motor current excitation

V	=	motor voltage excitation

θm	=	motor angular position

θ1	=	load (brake) angular position

θ2	=	tachometer angular position

| EXPERIMENTAL SET - UP |  |  |  |
| --- | --- | --- | --- |
| Internal combustion engine |  |  | 1300 cc lead less oil |
| Dynamometer heat absorption |  |  | water cooled |
| Internal combustion heat absorption |  |  | heat exchanger |
| Lubricating oil heat absorption |  |  | heat exchanger |
| Dynamometer casing | swing in trunion bearings, universal joints |  |  |
| Measurements |  |  | torque, angular velocity |
| Torsional strain |  |  | resistance strain gauges |
| Dynamometer speed |  |  | ac tachometer |
| Cooling water temperature |  |  | thermistor |
| Lubricant temperature |  |  | thermistor |
| Safety circuits |  |  | shut down |
| Control |  |  | angular position (15 Hz) |
| Throttle control |  |  | step motor |
| Mechanical properties |  | inertia, viscous damping, compliance |  |

Table 1.  Experimental set-up

The dynamometer field current is controlled with a rise time of 4 ms. Closed loop torque and speed control permits marginally greater bandwidth (3).

STATIC and DYNAMIC TESTS

The hardware under test and simulation is presented. The plant presented consists of the engine, the coupling shaft and the dynamometer. Parameters, such as the throttle angle, torque, field current and the speed measurement are given.

The model is developed initially to represent the system under steady state conditions, about a chosen operating point in the middle of the torque and speed ranges (5).

The model under dynamic conditions, the parameters interactions and magnitudes are evaluated by structural and dynamics considerations. Thermodynamic and mechanical losses are due to reciprocating and rotating parts (6). Excess attenuation at high frequencies of the plant is due to the flux build-up (contributing first order lags) associated with the dynamometer rotor (4).

Sine wave, step, impulse and Pseudo Random Binary Signal excitation responses are studied. PRBS excites many frequencies simultaneously, its power is spread over the lower harmonics of the spectrum, but it introduces noise in the system spectrum identification results.

engine torque				dynamometer torque

120					     120

[N.m]					   [N.m]

90					      90

throttle servo input	      		   dynamometer field current

60					      60

30					      30

0	0   1000   2000	3000   4000   5000	  0    1000   2000   3000   4000  5000   [rpm]

Fig.  4.  Engine and dynamometer torque - speed characteristic

The throttle and dynamometer steady state transfer characteristics are presented, together with the throttle-speed frequency response of the engine (see fig. 4). The response of torque and speed to throttle and load perturbations are simulated (see fig. 8) (12).

ENGINE - DYNAMOMETER MODEL

The state variables for the electromechanical model of the engine - dynamometer system, gives a model with a valid structure throughout the torque / speed range of the engine, as:

dVfdtdVDdtdIsdt=−1CE.RE01Ls−1CE.RE.RS0−1CD.RD1CD.RD.RS−1Ls−1CE1CD1Ls−1CE.RE.RS.VfVDIs+1CE.RE01CE.RE.RS0−1CD1CD.RS.VEID

where,

VF	=	engine speed			VD	=	dynamometer speed

IS	=	shaft torque			VT	=	throttle servo input voltage

VE	=	engine source voltage (a function of throttle opening)

-1/RE 	= 	slope of the torque / speed curve

VL	=	load - dynamometer current amplifier input voltage

ID	=	dynamometer source current

IF	=	field current

The model (a first order system with a time delay proportional to the inverse of engine speed, i.e. the time taken to inhale and ignite a charge) decouples the engine flywheel from the shaft connecting it to the dynamometer and obtains a series of transfer functions, accepted over a range of mean speeds. The engine inertia is lumped, while the dynamometer presents a large rotor inertia (7).

The responses of shaft torque and dynamometer speed to inputs to the dynamometer field current control show poor agreement, while for throttle servo disturbance they show good agreement.

Torque [N.m]				engine [rpm]

80

11000

60					   9000

7000

40

5000

20					   3000

1000

0	 0  0.1 0.2 0.3 0.4 0.5 0.6 0.7 0.8	   0    1.0    2.0   3.0  4.0  5.0   6.0

load current [A]			throttle [ V]

Fig.  5.  Throttle and dynamometer steady state transfer characteristic

Difficulties are encountered in reproducing the curve in the high torque and low speed regime, where RE becomes negative. At low speeds with large throttle openings the slope of the characteristic changes sign. Stability is achieved by proportional control upon the dynamometer field current. The slope of the torque / speed curves remains substantially constant for the full range of the throttle settings.

The relations between VT and VE are given in fig.  5, showing that for low speed high torque regime it is substantially linear.

steady excitation		braking range

motor

inductive current

field

current				     inductive current

speed

Fig. 6.   Characteristic curve of electrodynamic brake

The slope of the torque speed characteristics of the engine alter significantly in certain regimes of operation (i.e. the parameters change, see fig. 6). Instabilities cause the engine to stall, while resonance imposes limits on rig performance; feedback speed control overcomes such instabilities (see fig. 2).

[ N . 1000 ]

8

Braking

power

6

4

2

1

speed

0	0	10	20	30	40	50	60	70	[ miles/hr]

Fig. 7. Electric motor train Braking power vs velocity during maximum braking

Fig.  8.   Linear simulation of engine - dynamometer system

The throttle servo suffers from velocity limiting, as an average filter. Perturbations of the ignition advance instead of the throttle angle, results in an improved impulse response for the shaft torque (8).

CONCLUSIONS

The model of an internal combustion -dynamometer and its response is given. Simulation offers obvious advantages for testing the system without the danger in damaging it by over speeding. The system time constant is moderately short.

REFERENCES

(1)	Chilikin, M, Electric Drive, MIR Publishers, Moscow, 1976

(2)	Electro-Craft Corp, DC Motors Speed Controls Servo Systems, USA, 1984

(3)	FROUDE Consine Ltd, Installation, Operation & Maintenance Procedures,

Instruction Manual IM 1232/2, AG Series Eddy-Current Dynamometers, May

1990, FROUDE Consine Ltd

(4)	FROUDE Consine Ltd, Instruction Manual IM 1290/1, Issue 1, 	TEXCEL

100 Aero Version 3 Direc Digital Control System, July 1995, 	FROUDE

Consine Ltd

(5)	KISTLER Instruments AG, Quartz, 4-Component Dynamometer, 	Operating

Instructions, 1992

(6)	MAGTROL Inc, Digital Dynamometers, for Precision Motor Analysis, 1992

(7)	OSE, Handbook of Analysis, Control and Regulation of Dynamic Brake 	of

Diesel Traction Machines MLW S 450-500 and ALCO S 9100, June 1981,

Greek Railways, Study Group, Athens 1981 (in Greek)

(8)	Pattas, K. N, Stamatellos, A. M, The Effect of Exhaust Throttling on the 	Diesel Operation Characteristics and Thermal Loading, SAE, 890399, 	February 27-March 3, 1989, Detroit, Michigan

(9)	Plint, M, Martyr, A, Engine Testing, Theory and Practice, Chapter 7: 	Choosing the Right Dynamometer, p. 100, Butterworth-Heinemann Ltd, 	1995

(10)	Plint, M, Martyr, A, Engine Testing, Theory and Practice, Chapter 8: 	Coupling the Engine to the Dynamometer, p. 116, Butterworth - Heinemann

Ltd, 1995

(11)	Siakavellas, N, Eddy Current Modelling in a Coupled Electromagnetic-	Mechanical Problem, COMPEL, The International Journal for Computation

and Mathematics in Electrical and Electronic Engineering, Vol. 15, No. 2,

1996, pp. 52-72, MCB University Press

(12)	Ventzas D, Automation I-II, Lecture Notes, TEI Athens, STEF, TIO, 	Athens 1990 (in Greek)